
SECTION 11160

LOADING DOCK EQUIPMENT

Display hidden notes to specifier. (Don't know how? Click Here) 

Copyright 2008 - 2019 ARCAT, Inc. - All rights reserved

** NOTE TO SPECIFIER **  Blue Giant Equipment Corporation; dock levelers, dock lifts, dock safety systems, dock seals and shelters, scissor lifts, material handling and dock accessories.
This Section is based on the products of Blue Giant Equipment Corp., which is located at:
410 Admiral Blvd.
Mississauga, ON, Canada L5T 2N6
Toll Free:  800-872-2583
Tel:  905-457-3900
Fax:  905-457-2313
Web Site:  www.BlueGiant.com
 [Click Here ] for additional information.
Blue Giant Equipment Corporation, with corporate World Headquarters in Ontario, Canada is a global leader in the manufacture and distribution of dock equipment, ground level dock lifts, dock safety products and industrial trucks. Quite simply, there is a Blue Giant solution for your application.

1.1 WHEEL CHOCKS
A. Wheel Chock: Blue Giant Interlock Chock, as manufactured by Blue Giant, Model ITC.
1. Type: Mechanical wheel chock with position sensors.  Comes with docking cradle and a Blue Genius Control or can be ordered in combination with other equipment.

2. Overall Body Dimensions (WxLxH):  15-1/2 x 8 x 33 inches (394 x 203 x 838 mm).

3. Function:

a. Interlock Chock:  8” Wheel chock with Ultrasonic cradle sensor capable of detecting when the chock is parked in the cradle home position, engaged with a wheel and when unit is in neither position.

b. Door Interlock Sensor: Provides door position status information to Control Panel.

c. Dock Interlock: Provides dock stored status information to Control Panel.

d. Mounting System:  Cradle mounted with 4 bolts 3/8” 3 inch tap screws beside the dock position.  The Interlock Chock rests on this cradle when not in use.
4. Operation:

a. Operating System:
1) Positioning:  Intelligent positioning system ensures proper communications to the external dock worker and operator inside the building.

2) Sensors:

a) Chock Position Sensor: sensors indicate when chock is stored in cradle or engaged with wheel

b) Dock Stored Sensor: sensor indicates when the dock is fully parked

c) Door Sensor: sensors indicate when door is fully open or fully closed.

3) Audible:

a) Built-in Chock audible alarm linked to Control Panel
4) Locking:  Security locking system pin provides an option to lock the chock to the cradle.

b. Control Station:  24VDC control, with touch buttons.

1) NEMA4X/IP65, wet and corrosion resistant, rated polycarbonate enclosure.
** NOTE TO SPECIFIER **  Delete Control Station options not required.

2) The Blue Genius Gold Series II:  Multiple touch button control station, monitors equipment and provides alerts and alarms. 

a) Dimensions (LxWxD):  11-1/4 x 7-1/2 x 5-1/2 inches (287 x 190 x 140 mm).

b) Onboard diagnostics and communications feature a fully accessible LCD and keypad with supervisory override features.
c) Onboard LED traffic lights for material handling.

d) Audible piezo:, touch buttons, alerts, and alarms.

e) STOP button provides operation sequence pause of Interlock Chock and interlocked components.
3) Blue Genius Gold Series III Control Panel:  Multiple touch-button control station with constant pressure DECK and LIP touch buttons. Interlocks dock leveler and chock functions.  Normally bundled with other product orders.
a) Dimensions (LxWxD):  11-1/4 x 7-1/2 x 5-1/2 inches (287 x 190 x 140 mm).
b) Onboard diagnostics and communications feature a fully accessible LCD and keypad with supervisory override features.
c) Onboard LED traffic lights for material handling.

d) Audible piezo:, touch buttons, alerts, and alarms.

e) STOP button provides operation sequence pause of Interlock Chock and interlocked components.

f) STOP button limits equipment motion when touched.
g) ESC button activates release function.

c. Lights Communication System:  Exterior LED Traffic Lights:  The housing is constructed to withstand exterior weather conditions with a gasket mounting base, conduit punch-out provisions, and head with two visors and colored lenses. The lights shall be LED 4 inches (102 mm) diameter of red and green color. Overall body 5 inches W x 10 inches L x 3 inches D (127 x 254 x 76 mm) in size. 3-wire Voltage: 12-24V AC/DC 150-75 mA Warranty:  Warranties are subject to standard limitations on liability. Consult manufacturer for full details on warranty information and product registration.
1) External Light. Alerts external dock worker or truck driver to status of Interlock Chock.
2) Internal Light. Alerts internal dock attendant to status of Interlock Chock.
3) Chock Light. Alerts external dock worker or truck driver. When Interlock Chock is in cradle home position lights on handle will be OFF. When engaged with a wheel handle lights will be red and when various action conditions exist handle light will flash white.
5. Rated Capacity:  Rated in compliance with OSHA 1910.26(d) and 1910.178(k)(m).
** NOTE TO SPECIFIER **  Select one, delete others or insert power requirements.
Consult manufacturer.

6. Power:  __ V, __ Amps, __ Phase.

a. 110/120 V, 60 Hz, single phase.
b. 208/240 V, 60 Hz, single phase.
c. 440/480 V, three phase.
d. 575 V, 60 Hz, three phase.
e. 380/415 V, 50 Hz, three phase.
7. Construction and Finish:

a. Wheel Chock:
1) Housing:  Extruded aluminum structural housing protects all components from damage from weather, dirt and debris, finish in powder coat safety yellow.
2) Saw toothed Flange: provides solid grip between chock and grade surface
3) Operator Handle:  Constructed of structural steel with two hand grips

4) Cable Management system:  Fiberglass flexible wand to support the data cable from control to chock

5) Submersible components, ultrasonic sensor system
6) Overall Dimensions (WxLxH):  15-1/2 x 33 x 13 inches (394 x 838 x 332 mm).

** NOTE TO SPECIFIER **  Delete optional features/accessories not required.
8. Accessories:  Provide the following standard accessories.

a. Security locking system pin.
11160-3 

